Section VII

TECHNICAL SPECIFICATIONS

Procuring Entity: City Government of Tabaco City

Contract ID No.: 011-08-2020-GOODS

Name of the Project: Supply & Delivery of One Unit Brand New Forklift

Location of the Project: CDRRMO, Tabaco City

Technical Specifications

Bidders must state here either “Comply” or any equivalent term in the column “Bidder’s Statement of Compliance” against each of the individual parameters of each “Specification”. Brand of the item offered, if applicable, shall likewise be indicated in the column provided.

| Item No. | Description | Bidder’s Statement of Compliance |
| --- | --- | --- |
|  | Brand New Forklift |  |
|  | Rated Power: 55HP |  |
|  | Transmission Type: Torque Converter |  |
|  | Brake Distance: (m) <6 |  |
|  | Service Brake: Hydraulic Pedal Operation |  |
|  | Parking Brake: Mechanical-Hand Operation |  |
|  | Tire Type: Pneumatic |  |
|  | Tire Size (Front/Rear): 7.00-12/6.00-9 |  |
|  | Overall Height fork raised (with backrest): (mm) 4080 |  |
|  | Max lifting height: (mm) 3000, Free Length: (mm) 1070 |  |
|  | Overall Height (fork lowered, upright): (mm) 2095 |  |
|  | Overall width: (mm) 40, Fork width: (mm) 100 |  |
|  | Lateral Fork (outside fork): (mm) 1040 |  |
|  | Overhead guard clearance (from seat face): (mm) 1000 |  |
|  | Front overhang: (mm) 451, Rear overhang: (mm) 470 |  |
|  | Wheelbase: (mm) 1640, Tread (front tires): (mm) 970 |  |
|  | Tread (rear tires): (mm) 970 |  |
|  | Min. turning radius: (mm) 2425 |  |
|  | Min. under clearance (bottom of mast): (mm) 110 |  |
|  | Titling angles (forward/backward): (°) 6/12 |  |
|  | Min. Intersecting aisle: (mm) 2050 |  |
|  | Load Capacity: (kg) 2000 |  |
|  | Load Center: (mm) 500, Mast Type: 2 stage |  |
|  | Lifting speed (with load/without load): (mm/s) 400/600 |  |
|  | Lowering Speed (with load/without load): (mm/s) <420/320 |  |
|  | Max driving speed (with load/without load): (km/h) 17/19 |  |
|  | Max. Grade ability: (%) 20 |  |
|  | Weight distribution with load (front/rear): (%) 80/20 |  |
|  | Weight distribution without load (front/rear): 35/65 |  |
|  | Battery voltage / Capacity: (v/Ah) 90 |  |
|  | Total weight: (kg) 3350 |  |
|  | Make: preferably US, Japan, or UK made |  |
|  | Warranty: at least 3 years |  |

I hereby certify to comply with all the above Technical Specifications.

Name of the Company/Bidder		     Signature over Printed Name 	         Date

of Authorized Representative